PID Controller Parameters:

C =
 
             1          
  Kp + Ki * --- + Kd * s
             s          

  with Kp = 10.3, Ki = 13.1, Kd = 1.93
 
Continuous-time PID controller in parallel form.
Model Properties

========== PERFORMANCE METRICS ==========
 
--- OPEN LOOP RESPONSE ---
Rise Time        : 0.6903 sec
Settling Time    : 3.7352 sec
Overshoot        : 20.79 %
Undershoot       : -0.00 %
Peak Value       : 0.2416
Peak Time        : 1.5658 sec
 
--- CLOSED LOOP WITHOUT PID ---
Rise Time        : 0.6039 sec
Settling Time    : 3.4322 sec
Overshoot        : 24.50 %
Undershoot       : -0.00 %
Peak Value       : 0.2075
Peak Time        : 1.4276 sec
 
--- CLOSED LOOP WITH PID ---
Rise Time        : 0.4731 sec
Settling Time    : 2.4839 sec
Overshoot        : 3.35 %
Undershoot       : -0.00 %
Peak Value       : 1.0335
Peak Time        : 0.8531 sec